This demo is a Python (64-bit) example for Tashan tactile sensors.
 It can read the raw channel values and 3D force values from the sensors.

# Getting Started

## Install the CH341 Driver

### Windows

Run CH341PAR.exe directly and follow the prompts to install.

### Linux

Follow the instructions in the CH341PAR_LINUX/README.md file to install the driver.

## Linux Environment Setup

For Linux users, you need to add the CH341 device to the udev rules (so that normal users can access it),
 or run the program with sudo privileges (not recommended).

### Add User to the dialout Group

Check members of the dialout group. If the current user is not included, add the user to the group.
 Replace "john" with your actual username.

# Check members of the dialout group

getent group dialout

# Add user 'john' to the dialout group

sudo usermod -a -G dialout john

### Create a Rules File

Create a new rule file (e.g., ch341.rules) in /etc/udev/rules.d/ and add:

SUBSYSTEM=="usbmisc", ATTRS{idVendor}=="1a86", ATTRS{idProduct}=="5512", MODE="0666", GROUP="dialout"

- idVendor and idProduct are the vendor ID and product ID of the CH341 device.

- You can use the lsusb command to find these IDs.

### Reload Rules

sudo udevadm control --reload-rules

sudo udevadm trigger

# Run the Demo

## Power On

The startup time may vary depending on the firmware version of the sensor.
 It is recommended to wait 3 seconds after powering on the sensor before running the program.

## Run the Program from the Demo Root Directory

python test.py

### Example Output

capChannelDat=[3020838, 3006548, 2986382, 2929380, 2995152, 2992100, 2988970, 2996232, 2904946, 2947736, 2895000, 2842188, 2999608, 2922258, 2776718, 7950022]

nf[0] = 0.0

tf[0] = 0.0

tfDir[0] = 0

nf[1] = 0.0

tf[1] = 0.0

tfDir[1] = 0

sProxCapData[0] = 0

sProxCapData[1] = 0

mProxCapData[0] = 0

- capChannelDat = raw capacitive channel values

- nf, tf, tfDir = normal force, tangential force, and tangential force direction of one 3D force group

- sProxCapData = self-capacitive proximity values

- mProxCapData = mutual-capacitive proximity values

## Configure Number of Connected Fingers

The finger connection board supports up to 5 sensors.
 If fewer than 5 are connected, it may affect the read frequency.

Modify the following macro definition to set the number of connected sensors:

DEF_MAX_FINGER_NUM = 2  # Number of connected fingers, max = 5

## I2C Interface Order and Address Definition

In this demo, the I2C address corresponds to the fixed interface position on the adapter board.
 The mapping between I2C addresses and I2C interface positions is as follows:

If the sensor inserted does not match the predefined I2C address,
 the initialization process will force update the sensor’s I2C address to the predefined value.

def checkSensor(self):

# Read the current sensor address using broadcast,

# then update the I2C address to match the PCA index

addrRead = self.snsCmd.getAddr(0)

if addrRead != self.pcaIdx:

if self.pcaIdx != self.snsCmd.setAddr(addrRead, self.pcaIdx):

logger.error(f"set addr false, setaddr={self.pcaIdx}")

return False

else:

addrRead = self.pcaIdx

logger.info(f"update iic addr, new addr ={addrRead}")